NEXABANK ENTERPRISE NETWORK

LAB 09 — NAT & Internet Access (PAT)

# Lab Documentation Card

LAB NUMBER: 09

LAB TITLE: NAT & Internet Access (PAT)

DATE: 10 May 2026

SCENARIO: NexaBank HO — 500 employees accessing internet through single public IP on R1

WHY WE DID IT: Internal 10.1.x.x addresses are private — internet cannot route to them. PAT translates all internal IPs to R1's single public IP. Hides internal topology from internet — security + efficiency.

WHAT WE BUILT: PAT configured on R1. Inside interface = Gig0/0/0, Outside interface = Gig0/0/1. ACL permits all 10.1.x.x traffic. All 5 VLANs tested — 100% success.

WHAT BROKE: Nothing this lab

WHAT SAVED IT: N/A

REAL LIFE ACTION: In production, public IP is assigned by ISP — not 10.9.1.1. Real bank gets dedicated public IP block from ISP for NAT pool.

ISO 27001 LINK: A.13.1 — Network Controls. NAT hides internal topology. Protects network architecture from external exposure.

NIST CSF LINK: PR.AC-5 — Network integrity. DE.CM-1 — Network monitored. NAT translations logged for security monitoring.

BANK RELEVANCE: Bangladesh Bank ICT Guidelines require internal network not exposed to public internet. NAT/PAT = first line of defense. Hides ATM gateway, core banking servers from direct attack.

TEST RESULT: PAT configured on R1 ✅ | All 5 VLANs → ISP 4/4 ✅ | NAT translations verified ✅ | Internal IPs hidden from ISP ✅

# Key Commands

- ip nat inside                                → marks interface as inside (LAN)

- ip nat outside                               → marks interface as outside (WAN)

- access-list 1 permit 10.1.0.0 0.0.255.255   → permit all 10.1.x.x for NAT

- ip nat inside source list 1 int Gig0/0/1 overload  → enable PAT

- show ip nat translations                     → verify NAT table

# 1. Why NAT Exists

Private IPs (10.x.x.x) are not routable on the public internet. The ISP has no idea where 10.1.10.2 is. NAT bridges this gap by translating private addresses to a public IP that the internet understands.

- Without NAT → Internal devices cannot reach internet. Private IPs dropped at ISP.

- With NAT → R1 translates private to public. ISP only sees R1's public IP.

# 2. NAT Types

- Static NAT   → one private = one public (1:1 mapping, expensive, not scalable)

- Dynamic NAT  → pool of public IPs assigned dynamically

- PAT (NAT Overload) → many private = ONE public, differentiated by port number. NexaBank uses this.

# 3. How PAT Works

PC1 (10.1.10.2) sends packet → R1 translates source IP to 10.9.1.1 with unique port → ISP receives from 10.9.1.1 → reply comes back → R1 translates back to 10.1.10.2 → PC1 receives reply. 500 devices differentiated by port number only.

# 4. Wildcard Mask

Wildcard mask is the opposite of subnet mask. Used in ACLs, NAT, and OSPF.

- 0 = must match exactly

- 1 = don't care, match anything

- Quick flip: 255→0, 0→255, 224→31, 192→63, 128→127

- Example: 10.1.0.0  0.0.255.255 = match any 10.1.x.x address

- Calculation: 255.255.255.255 minus subnet mask = wildcard mask

# 5. NAT Translation Table

Verify with: show ip nat translations

- Inside local  = real private IP of device (10.1.x.x)

- Inside global = public IP ISP sees (10.9.1.1)

- Outside local = destination as seen from inside

- Outside global = destination real public IP

- Port after colon = PAT port number (unique per session)

# 6. Security Benefits of NAT

- Internal topology hidden from internet

- Attackers cannot directly target internal devices

- Finance, Teller, ATM systems protected from direct exposure

- Bangladesh Bank ICT Guidelines compliant

- First line of network defense

# 7. NexaBank IP Reference

- CS1 → R1 link:  Network 10.1.1.0/30 | CS1: 10.1.1.2 | R1: 10.1.1.1

- R1 → ISP link:  Network 10.9.1.0/30 | R1: 10.9.1.1  | ISP: 10.9.1.2

- NAT pool: R1 WAN IP (10.9.1.1) used as public IP for all internal devices

- ACL permits: 10.1.0.0 0.0.255.255 (all HO devices)

# 8. Bank Relevance

- Bangladesh Bank requires internal network not exposed to public internet

- All internet traffic must pass through controlled gateway (R1)

- NAT translations logged for audit trail

- Single exit point enables monitoring and control

- Real world: production uses actual public IP from ISP (e.g. 103.x.x.x in Bangladesh)

- Unusual NAT translation volumes = security alert (possible data exfiltration)

NexaBank Enterprise Network Project | Asfi Ahmed | GitHub: NexaBank-Enterprise-Network