╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 06

LAB TITLE     : Spanning Tree Protocol (RSTP)

DATE          : Today

SCENARIO      : NexaBank HO redundant links

between access switches and

CS1 core switch

WHY WE DID IT : Redundant links prevent single

point of failure but create loops.

RSTP blocks loops while maintaining

redundancy — critical for bank

uptime requirements

WHAT WE BUILT : Dual uplinks from AS1, AS2, AS3

to CS1. RSTP configured on CS1

as Root Bridge. Failover tested.

KEY COMMANDS  :

spanning-tree mode rapid-pvst

spanning-tree vlan [id] priority 4096

show spanning-tree

show mac address-table

show interfaces status

WHAT WE LEARNED:

→ Broadcast storms caused by loops

→ RSTP blocks redundant links automatically

→ Root Bridge = reference point for network

→ CS1 must always be Root Bridge

→ Priority 4096 beats default 32768

→ RSTP converges in 1-3 seconds

→ In PT — configure RSTP on CS1 only

→ Failover tested and confirmed working

TROUBLESHOOTING:

→ RSTP mode conflict between switches

caused traffic loss in PT

→ Fixed by removing rapid-pvst from

access switches — CS1 only needed

ISO 27001 LINK: A.17.2 — Redundancy of

information processing facilities

RSTP ensures network availability

NIST CSF LINK : PR.PT-5 — Mechanisms to achieve

resilience requirements

BANK RELEVANCE: Bangladesh Bank requires high

availability for banking systems.

RSTP ensures NexaBank stays online

even during link failures.

3 second failover vs 50 second

STP = critical for transactions

TEST RESULT   : Dual uplinks configured ✅

CS1 elected Root Bridge ✅

Failover test passed ✅

Network stable after recovery ✅

LAB 06 — SPANNING TREE PROTOCOL (RSTP)

========================================

1. WHY STP/RSTP EXISTS

────────────────────────

Redundant links = good for failover

Redundant links = bad if they create loops

Loops = broadcast storms = network crash

Broadcast storm:

→ Broadcast packet enters loop

→ Duplicates exponentially

→ Millions of packets in milliseconds

→ Switches crash, network down

→ Bank transactions fail ❌

RSTP solution:

→ Detects loops automatically

→ Blocks one redundant port

→ Keeps it on standby

→ Activates if primary fails ✅

2. STP vs RSTP

───────────────

STP  → original protocol

convergence = 30-50 seconds

too slow for banks ❌

RSTP → rapid version

convergence = 1-3 seconds

industry standard today ✅

NexaBank uses RSTP

3. ROOT BRIDGE

───────────────

Reference point for entire network

All paths calculated relative to Root Bridge

Switch with lowest Bridge ID wins election

Bridge ID = Priority + MAC Address

Lower priority = wins election

Default priority = 32768 (all switches)

CS1 priority     = 4096 (manually set)

Why CS1 must be Root Bridge:

→ Most powerful device

→ Centre of topology

→ All traffic flows through it

→ Access switch as Root = suboptimal paths

Command:

spanning-tree vlan [id] priority 4096

4. PORT ROLES & STATES

────────────────────────

Port Roles:

Root port      → best path to Root Bridge

Designated     → forwarding port on segment

Alternate      → standby, blocked

Port States:

Forwarding → active, sending traffic (GREEN)

Blocking   → standby, no traffic (ORANGE)

Disabled   → completely off

5. RSTP COMMANDS

─────────────────

Enable RSTP:

spanning-tree mode rapid-pvst

Set Root Bridge:

spanning-tree vlan 1,10,20,30,40,50,60 priority 4096

Verify:

show spanning-tree

→ Check "This bridge is the root"

→ Check protocol = rstp

→ Check port roles and states

6. PACKET TRACER NOTE

──────────────────────

Configure RSTP on CS1 only!

Access switches inherit mode from CS1.

Configuring rapid-pvst on access switches

causes port state confusion in PT = traffic loss.

Real world → configure on ALL switches

PT         → CS1 only ✅

7. NEXABANK RSTP DESIGN

────────────────────────

CS1 = Root Bridge (priority 4096)

AS1, AS2, AS3 = non-root switches

Dual uplinks per access switch:

Gig0/1 → primary uplink (forwarding)

Gig0/2 → backup uplink (blocking/standby)

Failover:

Primary link fails → RSTP detects

Standby activates → 1-3 seconds

Network restored → zero manual intervention

8. BROADCAST STORM CAUSES

───────────────────────────

1. RSTP disabled on a switch

2. Unmanaged switch connected by staff

3. Accidental cable loop

4. RSTP misconfiguration

Investigation commands:

show processes cpu     → confirm high CPU

show spanning-tree     → check topology changes

show interfaces trunk  → spot rogue connections

Physical inspection    → find looped cables

9. ISO 27001 & NIST LINKS

───────────────────────────

ISO 27001 → A.17.2 Redundancy of information

processing facilities

NIST CSF  → PR.PT-5 Mechanisms to achieve

resilience requirements

10. BANK RELEVANCE

───────────────────

Bangladesh Bank requires:

→ High availability for banking systems

→ Network redundancy documentation

→ Failover procedures documented

RSTP directly supports:

→ 99.99% uptime requirement

→ Transaction continuity

→ Zero manual intervention on link failure