╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 08

LAB TITLE     : Router Configuration & WAN

DATE          : Today

SCENARIO      : NexaBank HO router R1 configured

connecting internal LAN to ISP

WHY WE DID IT : CS1 handles internal routing but

cannot connect to WAN directly.

R1 is the gateway between NexaBank

internal network and internet.

WHAT WE BUILT : R1 configured with LAN and WAN

interfaces. Default routes on CS1

and R1. Static route on R1 for

internal network. End to end

connectivity from PC1 to ISP.

KEY COMMANDS  :

interface GigabitEthernet0/0/0

ip address [ip] [mask]

no shutdown

no switchport

ip route 0.0.0.0 0.0.0.0 [next-hop]

ip route [network] [mask] [next-hop]

show ip interface brief

show ip route

WHAT WE LEARNED:

→ ISR4331 uses Gig0/0/0 notation

→ no switchport converts L2 to L3 port

→ Default route = gateway of last resort

→ Static route = specific network path

→ CS1 handles internal, R1 handles external

→ /30 used for point-to-point links

→ Return path equally important as forward

WHAT BROKE    : None this lab

WHAT SAVED IT : N/A

REAL LIFE ACTION: In production, default routes

and static routes are replaced

with dynamic routing (OSPF)

for automatic route updates.

We cover this in Lab 10.

ISO 27001 LINK: A.13.1 — Network Controls

Routing controls traffic flow

between internal and external

networks

NIST CSF LINK : PR.AC-5 — Network integrity

protected through controlled

routing policies

BANK RELEVANCE: Every bank has a dedicated router

as WAN gateway. Bangladesh Bank

requires documented routing policy.

R1 is the single controlled exit

point — easier to monitor and

secure all outbound traffic.

TEST RESULT   : R1 interfaces up/up ✅

CS1 → R1 ping 100% ✅

PC1 → ISP ping 100% ✅

End to end connectivity ✅

LAB 08 — ROUTER CONFIGURATION & WAN

======================================

1. ROUTER ROLE IN ENTERPRISE

──────────────────────────────

CS1 = internal brain (VLAN routing)

R1  = external gateway (WAN routing)

Handoff point = 10.1.1.0/30 link

CS1 handles:

→ Inter-VLAN routing

→ Internal 10.1.x.x traffic

→ Layer 3 switching

R1 handles:

→ WAN connectivity

→ Internet traffic

→ NAT (next lab)

→ VPN (later lab)

2. ISR 4331 INTERFACE NOTATION

────────────────────────────────

ISR 4331 uses triple notation:

GigabitEthernet0/0/0 → first interface

GigabitEthernet0/0/1 → second interface

GigabitEthernet0/0/2 → third interface

Different from Cat switches (Gig0/1)

Always check device type before typing

3. NO SWITCHPORT COMMAND

──────────────────────────

Cat3650 ports = Layer 2 by default

For routed links needing IP address:

→ must convert to Layer 3 first

no switchport → converts L2 to L3

After this → can assign IP address

Use when:

→ Connecting core switch to router

→ Port needs its own IP address

→ Routed link between devices

4. ROUTING CONCEPTS

─────────────────────

Default route:

→ 0.0.0.0 0.0.0.0 = matches everything

→ Gateway of last resort

→ Used when no specific route exists

→ Points to next hop

Static route:

→ Specific network manually configured

→ One direction only

→ Does not update automatically

NexaBank routes:

CS1 default → ip route 0.0.0.0 0.0.0.0 10.1.1.1

(unknown traffic → R1)

R1 default  → ip route 0.0.0.0 0.0.0.0 10.9.1.2

(unknown traffic → ISP)

R1 static   → ip route 10.1.0.0 255.255.0.0 10.1.1.2

(internal traffic → CS1)

5. NEXABANK IP SCHEME — WAN LINKS

────────────────────────────────────

CS1 → R1 link:

Network   : 10.1.1.0/30

CS1       : 10.1.1.2

R1        : 10.1.1.1

R1 → ISP link:

Network   : 10.9.1.0/30

R1        : 10.9.1.1

ISP       : 10.9.1.2

6. PACKET FLOW

───────────────

LAN to Internet:

PC → AS → CS1 → R1 → ISP

Internet to LAN:

ISP → R1 → CS1 → AS → PC

CS1 decision:

→ knows 10.1.x.x = internal = route directly

→ unknown = default route = send to R1

R1 decision:

→ knows 10.1.x.x = internal = send to CS1

→ unknown = default route = send to ISP

7. WHY /30 FOR P2P LINKS

──────────────────────────

Point-to-point = only 2 devices

/30 = exactly 2 usable addresses

/24 = 254 usable = 252 wasted

Efficient IP planning = professional habit

Always use /30 for router-to-router links

8. ISO 27001 & NIST LINKS

───────────────────────────

ISO 27001 → A.13.1 Network Controls

Routing controls traffic between

internal and external networks

NIST CSF  → PR.AC-5 Network integrity

protected through routing policy

9. BANK RELEVANCE

──────────────────

Bangladesh Bank requires:

→ Single controlled WAN exit point

→ Documented routing policy

→ All outbound traffic monitored

→ Router as security checkpoint

R1 = single exit point for NexaBank

= easier to monitor all traffic

= security policy applied here

= NAT configured here (next lab)

10. WHAT BROKE / SAVED / REAL LIFE

─────────────────────────────────────

WHAT BROKE:

→ Nothing major this lab

WHAT SAVED IT:

→ N/A

REAL LIFE ACTION:

→ Static routes replaced with OSPF

in production for automatic updates

→ Covered in Lab 10

→ Static routes used for simple

or backup paths only