LAB 10

OSPF Multi-Area

# Lab Documentation Card

LAB NUMBER: 10

LAB TITLE: OSPF Multi-Area

DATE: 15 May 2026

SCENARIO: NexaBank HO — replacing static routing with OSPF dynamic routing between CS1 and R1

WHY WE DID IT: Static routing is manual, rigid and doesn't adapt to failures. OSPF automatically discovers routes, shares them with neighbours and self-heals when links fail. Critical for enterprise bank uptime.

WHAT WE BUILT: OSPF configured on R1 and CS1. All 6 VLANs advertised automatically. Default route advertised via OSPF. Failover tested — network self-healed in seconds.

WHAT BROKE: Nothing this lab

WHAT SAVED IT: N/A

REAL LIFE ACTION: In production with multiple branch routers, OSPF areas would be used — Area 0 for HO backbone, Area 1 for BR1, Area 2 for BR2. ABR routers sit between areas summarising routes.

ISO 27001 LINK: A.13.1 — Network Controls. OSPF ensures network resilience and automatic failover.

NIST CSF LINK: PR.PT-5 — Resilience mechanisms. RS.RP-1 — Recovery plan executed during events.

BANK RELEVANCE: Bangladesh Bank requires high availability. OSPF ensures automatic route recovery — no manual intervention at 3am when links fail.

TEST RESULT: OSPF neighbours FULL state ✅ | All VLANs learned via OSPF ✅ | Default route advertised ✅ | Failover test passed ✅ | PC1→ISP 4/4 ✅

# Key Commands

- router ospf 1                              → start OSPF process 1

- network [ip] [wildcard] area [id]          → advertise network in area

- default-information originate              → advertise default route via OSPF

- no ip route [network] [mask] [next-hop]    → remove static route

- show ip ospf neighbor                      → verify neighbour state

- show ip route                              → verify routing table

# Notes

## 1. Why OSPF Replaces Static Routing

- Static routing = manual, rigid, human dependent

- Static routing = if link fails, traffic keeps going to dead route

- OSPF = dynamic, automatic, self healing

- OSPF = routers discover neighbours and share routing info automatically

- OSPF = if link fails, routes removed and alternate path found instantly

- Analogy: Static = paper map | OSPF = Google Maps

## 2. OSPF Benefits

- Automatic routing — no manual config after initial setup

- Redundancy — adapts to link failures automatically

- Shortest path — always picks lowest cost path (Dijkstra algorithm)

- Scalability — works for 3 routers or 500 routers equally well

- Fast convergence — seconds not minutes

## 3. OSPF Areas

- Area = group of routers sharing routing info internally

- Area 0 = Backbone (mandatory, always exists)

- All other areas MUST connect to Area 0

- ABR = Area Border Router = sits between two areas

- ABR summarises routes between areas — reduces overhead

- Analogy: Area = class section | ABR = CR connecting sections

- NexaBank: Area 0 = HO | Area 1 = BR1 | Area 2 = BR2 (future labs)

## 4. Wildcard Masks for OSPF

- /30 → 0.0.0.3

- /28 → 0.0.0.15

- /27 → 0.0.0.31

- /26 → 0.0.0.63

- /25 → 0.0.0.127

- /24 → 0.0.0.255

- /16 → 0.0.255.255

- Calculation: 255.255.255.255 minus subnet mask = wildcard

## 5. OSPF Configuration — NexaBank

- R1:  router ospf 1

- network 10.1.1.0 0.0.0.3 area 0   (CS1-R1 link)

- network 10.9.1.0 0.0.0.3 area 0   (R1-ISP link)

- ip route 0.0.0.0 0.0.0.0 10.9.1.2 (default to ISP)

- default-information originate      (advertise default via OSPF)

- CS1: router ospf 1

- network 10.1.1.0  0.0.0.3   area 0  (CS1-R1 link)

- network 10.1.10.0 0.0.0.31  area 0  (Management)

- network 10.1.20.0 0.0.0.63  area 0  (IT)

- network 10.1.30.0 0.0.0.127 area 0  (Finance)

- network 10.1.40.0 0.0.0.31  area 0  (HR)

- network 10.1.50.0 0.0.0.63  area 0  (Tellers)

- network 10.1.60.0 0.0.0.15  area 0  (DMZ)

## 6. OSPF Route Codes in Routing Table

- O      = OSPF learned route

- O*E2   = OSPF external default route (imported from outside OSPF)

- C      = directly connected

- S      = static route

- S*     = static default route

- [110/2] = [administrative distance / metric (cost)]

- Gateway of last resort = default route destination

## 7. OSPF Neighbour States

- FULL = fully converged, sharing all routing info (what we want)

- LOADING = still exchanging routing database

- DOWN = neighbour lost, link failed or misconfigured

## 8. OSPF Neighbour DOWN Causes

- Physical — cable disconnected or port shutdown

- Area mismatch — CS1 area 0, R1 area 1 by mistake

- Hello/Dead timer mismatch between routers

- OSPF process stopped — no router ospf 1

- IP misconfiguration on link

- Investigate: show ip ospf neighbor | show ip interface brief

## 9. default-information originate

- R1 has static default route → 0.0.0.0/0 via ISP

- This route is OUTSIDE OSPF domain

- default-information originate → imports it INTO OSPF

- CS1 receives O*E2 route automatically

- All internal devices get internet access via OSPF

- Without this → CS1 has no default route → no internet

## 10. ISO 27001 & NIST Links

- ISO 27001 → A.13.1 Network Controls — OSPF ensures resilient routing

- ISO 27001 → A.17.2 Redundancy — automatic failover without manual work

- NIST CSF  → PR.PT-5 Resilience mechanisms implemented

- NIST CSF  → RS.RP-1 Recovery plan executed during network events

## 11. Bank Relevance

- Bangladesh Bank requires high availability for banking systems

- OSPF self-heals at 3am — no engineer intervention needed

- Transaction continuity maintained during link failures

- OSPF convergence = seconds vs static = manual fix = minutes/hours

- Every major bank in Bangladesh runs OSPF on their core network

NexaBank Enterprise Network Project | Asfi Ahmed | GitHub: NexaBank-Enterprise-Network