╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 03

LAB TITLE     : Device Selection & Rack Design

DATE          : Today

SCENARIO      : NexaBank HO physical infrastructure

design and device selection

WHY WE DID IT : Real engineers select and document

devices before building anything

WHAT WE BUILT : HO rack diagram with 10 devices

across 10U of rack space

KEY COMMANDS  : None — design phase

WHAT WE LEARNED:

→ Device roles (router, firewall, L2/L3 switch)

→ Real world vs Packet Tracer equivalents

→ Rack unit (U) measurement system

→ Device selection justification

PACKET TRACER : ISR4331, ASA5505, Cat3650,

EQUIVALENTS   Cat2960, Generic Server

ISO 27001 LINK: A.11 — Physical & Environmental

Security

NIST CSF LINK : PR.AC-2 — Physical access managed

BANK RELEVANCE: Bangladesh Bank requires documented

physical infrastructure for audits

TEST RESULT   : Rack diagram exported ✅

LAB 03 — DEVICE SELECTION & RACK DESIGN

=========================================

1. DEVICE ROLES

────────────────

Router   → Connects network to ISP/WAN

Handles NAT, VPN, routing protocols

Firewall → Sits between router and network

Handles ACLs, DMZ zones, VPN tunnels

Core SW  → Layer 3 — connects all switches together

Routes between VLANs (no separate router needed)

Access SW→ Layer 2 — connects end devices only

One per floor or department

Server   → Hosts services (web, mail, ATM gateway)

Rack mounted, enterprise grade

2. NEXABANK DEVICE SELECTION

──────────────────────────────

REAL WORLD              PACKET TRACER EQUIVALENT

─────────────────────────────────────────────────

Cisco ISR 4331      →   Cisco ISR 4331 (exact match)

Cisco ASA 5506-X    →   Cisco ASA 5505 (closest in PT)

Cisco Cat 3650      →   Cisco Cat 3650 (exact match)

Cisco Cat 2960-X    →   Cisco Cat 2960 (exact match)

Dell PowerEdge R340 →   Generic Server (no Dell in PT)

Note: ASA 5505 used in PT — same concept,

older model. Document this in every lab.

3. WHY EACH DEVICE WAS SELECTED

─────────────────────────────────

ISR 4331  → Mid-enterprise router, supports VPN,

NAT, OSPF. Perfect for NexaBank size.

ASA 5506-X→ Bank grade firewall. Handles DMZ,

ACL, VPN. Bangladesh Bank compliant.

Cat 3650  → Layer 3 core switch. Routes between

VLANs. High throughput for backbone.

Cat 2960  → Layer 2 access switch. Simple,

reliable, cost effective for floors.

PowerEdge → Industry standard rack server.

Hot-swappable, enterprise reliable.

4. DEVICE SELECTION RULES

───────────────────────────

Router   → must support WAN, VPN, NAT, routing

Firewall → must support DMZ, ACL, VPN tunnels

Core SW  → MUST be Layer 3 (inter-VLAN routing)

Access SW→ Layer 2 is fine (just end devices)

Servers  → rack mounted, enterprise grade

5. HEAD OFFICE RACK LAYOUT

────────────────────────────

U1  — Cisco ISR 4331      Core Router

U2  — Cisco ASA 5505      Firewall

U3  — Cisco Cat 3650      Core Switch

U4  — Cisco Cat 2960      Access Switch AS1 (Floor 1)

U5  — Cisco Cat 2960      Access Switch AS2 (Floor 2)

U6  — Cisco Cat 2960      Access Switch AS3 (Floor 3)

U7  — Cisco Cat 2960      DMZ Switch

U8  — Dell PowerEdge R340 WEB-SVR

U9  — Dell PowerEdge R340 ATM-GW

U10 — Dell PowerEdge R340 MAIL-SVR

U = Rack Unit (standard measurement)

Devices mount top to bottom by priority

Router and firewall always at top (WAN first)

6. RACK DESIGN RULES

──────────────────────

→ Router at top — first point of entry

→ Firewall directly below router

→ Core switch below firewall

→ Access switches below core switch

→ Servers at bottom

→ Each device labelled with U number and name

7. ISO 27001 & NIST LINKS

───────────────────────────

ISO 27001 → A.11 Physical & Environmental Security

Documents physical device placement

and access controls

NIST CSF  → PR.AC-2 Physical access is managed

and protected

8. BANK RELEVANCE

──────────────────

Bangladesh Bank requires:

→ Documented physical infrastructure

→ Device inventory for audits

→ Rack diagrams for compliance inspections

→ Justification for every device selected