LAB 12

DMZ Architecture

# Lab Documentation Card

LAB NUMBER: 12

LAB TITLE: DMZ Architecture

DATE: 21 May 2026

SCENARIO: NexaBank HO — isolating public facing servers (WEB-SVR, ATM-GW, MAIL-SVR) in a dedicated DMZ zone, protected from internal LAN

WHY WE DID IT: Public facing servers must be accessible from internet but must never reach internal departments. A compromised web server must not give attackers access to Finance or HR. DMZ contains the blast radius to the public zone only.

WHAT WE BUILT: DMZ-SW1 configured with VLAN 60. WEB-SVR, ATM-GW, MAIL-SVR connected with IPs assigned. ACL_DMZ blocks DMZ from reaching all internal VLANs while permitting internet access.

WHAT BROKE: Initially DMZ could reach internal network — expected, no ACL existed yet for VLAN 60

WHAT SAVED IT: Added ACL_DMZ inbound on VLAN 60 interface — blocked all DMZ to internal traffic

REAL LIFE ACTION: In production, ATM-GW needs specific access to core banking server. Add permit rule BEFORE deny rules using host keyword and specific port (443) — least privilege principle. Never permit entire DMZ subnet to internal.

ISO 27001 LINK: A.13.1 — Network Controls | A.13.2 — Information transfer | DMZ enforces network boundary between trusted and untrusted zones

NIST CSF LINK: PR.AC-5 — Network integrity | DE.CM-1 — Network monitored | PR.DS-5 — Protections against data leaks

BANK RELEVANCE: Bangladesh Bank ICT Guidelines mandate DMZ architecture for banks running public facing services. ATM networks, internet banking portals and email servers must be isolated from core banking systems.

TEST RESULT: DMZ servers → Internet 4/4 ✅ | DMZ → Finance blocked ✅ | DMZ → Management blocked ✅ | ACL_DMZ verified ✅

# Key Commands

- hostname DMZ-SW1                            → name the switch

- vlan 60 / name DMZ                          → create DMZ VLAN

- interface Gig0/1 / switchport mode trunk    → trunk to CS1

- int range fa0/1-3 / switchport access vlan 60 → assign server ports

- ip access-list extended ACL_DMZ             → create DMZ ACL

- deny ip 10.1.60.0 0.0.0.15 10.1.x.0 0.0.0.x → block DMZ to internal

- permit ip any any                           → allow internet access

- interface vlan 60 / ip access-group ACL_DMZ in → apply ACL

- permit tcp host [DMZ-IP] host [internal-IP] eq 443 → specific server access

# Notes

## 1. What is a DMZ

- DMZ = Demilitarised Zone — isolated middle zone between internet and internal LAN

- Sits between untrusted (internet) and trusted (internal LAN) zones

- Hosts public facing servers — web, email, ATM gateway

- Analogy: Bank reception area — public can enter, vault is off limits

- Even if DMZ server is compromised — internal LAN stays safe

## 2. DMZ vs VLAN

- VLAN = logical segmentation INSIDE internal network (trusted zone)

- DMZ = architectural separation BETWEEN internet and internal (untrusted zone)

- VLAN servers still part of internal network — one misconfigured ACL = breach

- DMZ structurally isolated — defence in depth even if ACLs fail

- Bangladesh Bank mandates proper DMZ — not just a VLAN — for public services

## 3. NexaBank DMZ Design

- VLAN 60 = DMZ subnet — 10.1.60.0/28

- DMZ-SW1 = dedicated switch for DMZ servers

- WEB-SVR  → 10.1.60.2 (public website)

- ATM-GW   → 10.1.60.3 (ATM transaction gateway)

- MAIL-SVR → 10.1.60.4 (email server)

- Gateway  → 10.1.60.1 (CS1 SVI VLAN60)

- DMZ → internet = permitted

- DMZ → internal = blocked by ACL_DMZ

## 4. ACL_DMZ Rules

- deny ip 10.1.60.0 0.0.0.15 10.1.10.0 0.0.0.31  → DMZ cannot reach Management

- deny ip 10.1.60.0 0.0.0.15 10.1.20.0 0.0.0.63  → DMZ cannot reach IT

- deny ip 10.1.60.0 0.0.0.15 10.1.30.0 0.0.0.127 → DMZ cannot reach Finance

- deny ip 10.1.60.0 0.0.0.15 10.1.40.0 0.0.0.31  → DMZ cannot reach HR

- deny ip 10.1.60.0 0.0.0.15 10.1.50.0 0.0.0.63  → DMZ cannot reach Tellers

- permit ip any any                                → internet access allowed

- Applied inbound on interface vlan 60

## 5. Real World — ATM-GW to Core Banking

- Problem: ATM-GW needs to reach core banking server to process transactions

- Solution: Add specific permit BEFORE deny rules

- permit tcp host 10.1.60.3 host [core-banking-IP] eq 443

- Only ATM-GW (not entire DMZ) can reach only core banking server on port 443

- Least privilege principle — minimum access needed, nothing more

- Rule order critical — specific permit FIRST, broad deny AFTER

- host keyword = single specific IP address

## 6. Least Privilege Principle

- Give minimum access needed — nothing more

- Never permit entire subnet when one specific IP is enough

- Never permit all ports when one specific port is enough

- Never permit all protocols when TCP is enough

- Reduces attack surface significantly

- Bangladesh Bank ICT Guidelines require least privilege for all access

## 7. Blast Radius Concept

- Blast radius = how much damage an attacker can cause if they breach one system

- Without DMZ: breach web server = access to entire internal LAN = catastrophic

- With DMZ: breach web server = stuck in DMZ = internal LAN safe

- DMZ minimises blast radius to public zone only

- ACLs further restrict even within DMZ

- Defence in depth = multiple layers of protection

## 8. Network Traffic Flow

- Internet → WEB-SVR: permitted (public website access)

- Internet → ATM-GW: permitted (ATM transactions)

- WEB-SVR → Internet: permitted (outbound allowed)

- WEB-SVR → Finance: BLOCKED by ACL_DMZ

- WEB-SVR → Management: BLOCKED by ACL_DMZ

- ATM-GW → Core Banking: specific permit only (real world)

- Internal LAN → DMZ: permitted (admins can manage servers)

## 9. ISO 27001 & NIST Links

- ISO 27001 → A.13.1 Network Controls — DMZ enforces network boundary

- ISO 27001 → A.13.2 Information transfer — controls data flow between zones

- ISO 27001 → A.14.1 Security requirements — security built into architecture

- NIST CSF  → PR.AC-5 Network integrity protected through DMZ isolation

- NIST CSF  → DE.CM-1 Network monitored — DMZ traffic logged

- NIST CSF  → PR.DS-5 Protections against data leaks from internal to DMZ

## 10. Bank Relevance

- Bangladesh Bank mandates DMZ for all banks with public facing services

- ATM networks must be isolated from core banking — separate zone required

- Internet banking portal in DMZ — never on internal LAN

- Email server in DMZ — phishing attacks contained to DMZ only

- Regular DMZ security audits required by Bangladesh Bank

- Compromised DMZ server must not lead to core banking data breach

## 11. What Broke / Saved / Real Life

- WHAT BROKE: DMZ could reach internal before ACL_DMZ was applied — expected

- WHAT SAVED IT: ACL_DMZ applied inbound on VLAN 60 — blocked all DMZ to internal

- REAL LIFE ACTION: ATM-GW needs specific access to core banking server.

- Add: permit tcp host 10.1.60.3 host [core-banking] eq 443 BEFORE deny rules.

- Always use host keyword for specific IPs — never permit entire DMZ subnet inward.

- Document all DMZ access rules in security policy for Bangladesh Bank audit.

NexaBank Enterprise Network Project | Asfi Ahmed | GitHub: NexaBank-Enterprise-Network