LAB 13

DHCP Server & DHCP Snooping

# Lab Documentation Card

LAB NUMBER: 13

LAB TITLE: DHCP Server & DHCP Snooping

DATE: 27 May 2026

SCENARIO: NexaBank HO — automating IP assignment across all VLANs using CS1 as DHCP server, and implementing DHCP Snooping to prevent rogue DHCP attacks

WHY WE DID IT: Manual IP assignment for 500 employees is unmanageable. DHCP automates this. Without DHCP Snooping, an attacker can plug in a rogue DHCP server and redirect all bank traffic through their device — Man in the Middle attack.

WHAT WE BUILT: DHCP pools on CS1 for VLANs 10,20,30,40,50. Excluded first 5 IPs per subnet. All 5 PCs received correct IPs automatically. DHCP Snooping documented for production deployment.

WHAT BROKE: DHCP Snooping caused APIPA on PCs — PT limitation with EtherChannel (port-channel trust not supported). DHCP Snooping removed from PT lab.

WHAT SAVED IT: Removed DHCP Snooping from access switches — DHCP immediately restored. PT limitation documented.

REAL LIFE ACTION: In production: configure DHCP Snooping on access switches, trust uplink ports including port-channel interfaces. This works on real Cisco hardware. See Section 8 for complete production deployment guide.

ISO 27001 LINK: A.9.1 — Access Control Policy | A.13.1 — Network Controls | DHCP Snooping prevents unauthorised network access via rogue DHCP

NIST CSF LINK: PR.AC-3 — Remote access managed | PR.AC-5 — Network integrity | DE.CM-1 — Network monitored

BANK RELEVANCE: Bangladesh Bank ICT Guidelines require protection against Layer 2 attacks. Rogue DHCP = Man in the Middle = bank transaction interception. DHCP Snooping is mandatory security control in banking networks.

TEST RESULT: All 5 VLANs DHCP working ✅ | Correct IPs assigned ✅ | Correct gateways ✅ | DHCP binding table verified ✅ | Ping to ISP 4/4 ✅

# Key Commands — DHCP Server

- ip dhcp excluded-address [start] [end]    → reserve IP range from DHCP pool

- ip dhcp pool [NAME]                       → create named DHCP pool

- network [subnet] [mask]                   → define pool subnet

- default-router [gateway IP]               → assign default gateway to clients

- dns-server [DNS IP]                       → assign DNS server to clients

- show ip dhcp binding                      → verify IP assignments and MACs

- show ip dhcp pool                         → verify pool configuration

- clear ip dhcp binding *                   → clear all DHCP bindings (prod only)

# Key Commands — DHCP Snooping (Production)

- ip dhcp snooping                          → enable DHCP snooping globally

- ip dhcp snooping vlan [id,id,id]          → enable on specific VLANs

- no ip dhcp snooping information option    → disable option 82 if needed

- interface [uplink port]                   → enter trusted uplink interface

- ip dhcp snooping trust                    → mark port as trusted

- show ip dhcp snooping                     → verify trusted/untrusted ports

- show ip dhcp snooping binding             → verify snooping binding table

# Notes

## 1. Why DHCP Exists

- Manual IP assignment for 500 employees = unmanageable and error prone

- Duplicate IPs from human error = network conflicts = devices go offline

- DHCP automates IP assignment — plug in device, get IP instantly

- IT team manages one DHCP server instead of 500 individual configs

- Lease system ensures IPs are recycled when devices leave network

## 2. DORA Process — How DHCP Works

- Step 1 DISCOVER → device broadcasts 'Is there a DHCP server here?'

- Step 2 OFFER    → DHCP server responds 'Yes! I'll give you 10.1.10.6'

- Step 3 REQUEST  → device says 'Yes please, I'll take 10.1.10.6'

- Step 4 ACK      → server confirms '10.1.10.6 is yours for lease period'

- DORA = Discover, Offer, Request, Acknowledge — memorise this

- All DISCOVER and REQUEST = broadcasts (goes to entire subnet)

- OFFER and ACK = unicast from server to client

## 3. DHCP Key Concepts

- DHCP Pool    = range of IPs available for automatic assignment

- Lease time   = how long device keeps the IP before renewal

- Excluded IPs = reserved addresses DHCP will never assign

- Default GW   = router/SVI address given to all clients

- DNS server   = name resolution server (8.8.8.8 = Google DNS)

- APIPA        = 169.254.x.x = self assigned when DHCP fails = problem!

## 4. Why We Excluded First 5 IPs

- .1 = SVI (default gateway) — always static, must never be assigned by DHCP

- .2 to .5 = reserved for future static devices (printers, IP phones, servers)

- If DHCP assigns .1 to a PC — gateway conflict — entire VLAN goes down

- Industry standard = exclude first 5-10 IPs per subnet

- Forward planning = leave room for infrastructure devices

## 5. NexaBank DHCP Configuration

- DHCP Server: CS1 (Core L3 Switch)

- VLAN10_MGMT  → 10.1.10.0/27  Excluded: .1-.5  First assignable: .6

- VLAN20_IT    → 10.1.20.0/26  Excluded: .1-.5  First assignable: .6

- VLAN30_FINANCE → 10.1.30.0/25 Excluded: .1-.5 First assignable: .6

- VLAN40_HR    → 10.1.40.0/27  Excluded: .1-.5  First assignable: .6

- VLAN50_TELLERS → 10.1.50.0/26 Excluded: .1-.5 First assignable: .6

- DMZ (VLAN60) → NO DHCP — servers always use static IPs

- DNS server: 8.8.8.8 (Google DNS) for all pools

## 6. Static vs DHCP — When to Use Which

- DHCP  → end user devices (PCs, laptops, mobile devices)

- Static → servers (web, mail, ATM gateway, core banking)

- Static → network devices (routers, switches, firewalls)

- Static → printers, IP phones, access points

- Rule: anything that needs a predictable fixed address = static

- Rule: anything temporary or user facing = DHCP

- If server IP changes → DNS breaks, ACLs break, firewall rules break = disaster

SECTION 7 — DHCP SNOOPING: COMPLETE PRODUCTION GUIDE

## 7A. What is DHCP Snooping

DHCP Snooping is a Layer 2 security feature built into Cisco switches that acts as a firewall between end devices and the DHCP server. It inspects all DHCP messages and enforces a trust model — only designated trusted ports can send DHCP server responses. All other ports are untrusted and their DHCP responses are dropped immediately.

## 7B. The Threat — Rogue DHCP Attack

- Attacker plugs laptop into any switch port in the bank

- Attacker runs rogue DHCP server software on their laptop

- Employee PC connects → broadcasts DHCP Discover

- Rogue server responds FASTER than legitimate CS1

- Employee PC accepts rogue offer:

- IP: 10.1.50.7 (looks legitimate)

- Gateway: 10.1.50.99 (ATTACKER'S LAPTOP!)

- ALL traffic from employee now flows through attacker's laptop

- Attacker sees: login credentials, banking transactions, internal data

- This is Man in the Middle (MitM) attack — silent, invisible, devastating

- Bangladesh Bank ICT Guidelines classify this as critical threat

## 7C. How DHCP Snooping Stops It

- Switch classifies every port as TRUSTED or UNTRUSTED

- TRUSTED ports = uplinks to legitimate DHCP server (CS1)

- UNTRUSTED ports = end device ports (PCs, printers, rogue devices)

- DHCP OFFER/ACK from UNTRUSTED port = immediately dropped

- Rogue server plugged into Fa0/1 → untrusted → OFFER blocked → attack fails

- Legitimate CS1 OFFER comes via trusted uplink → accepted → IP assigned correctly

- Snooping also builds binding table: IP + MAC + port + VLAN

- Binding table used by DAI (Lab 14) for ARP inspection

## 7D. Trust Model — NexaBank Design

- TRUSTED ports on AS1/AS2/AS3:

- GigabitEthernet0/1 → uplink to CS1 (trusted — DHCP comes from here)

- GigabitEthernet0/2 → uplink to CS1 (trusted — redundant uplink)

- Port-Channel1/2/3  → EtherChannel to CS1 (trusted — logical uplink)

- UNTRUSTED ports on AS1/AS2/AS3:

- FastEthernet0/1    → PC/end device (untrusted — no DHCP server here)

- FastEthernet0/2    → PC/end device (untrusted)

- All other Fa ports → untrusted by default

- CS1 itself does NOT need snooping — it IS the DHCP server

## 7E. Complete Production Configuration

Apply on AS1, AS2, AS3 — on real Cisco hardware:

- Step 1 — Enable DHCP Snooping globally:

- ip dhcp snooping

- Step 2 — Enable on specific VLANs:

- ip dhcp snooping vlan 10,20,30,40,50

- Step 3 — Disable Option 82 if not required:

- no ip dhcp snooping information option

- (Option 82 adds relay agent info — can cause issues if DHCP server

- doesn't support it. Disable unless explicitly required.)

- Step 4 — Trust uplink physical ports:

- interface GigabitEthernet0/1

- ip dhcp snooping trust

- exit

- interface GigabitEthernet0/2

- ip dhcp snooping trust

- exit

- Step 5 — Trust EtherChannel logical interface:

- interface port-channel 1

- ip dhcp snooping trust

- exit

- (On real hardware this works — PT limitation only)

- Step 6 — Verify configuration:

- show ip dhcp snooping

- show ip dhcp snooping binding

- show ip dhcp snooping statistics

## 7F. Verification Commands Explained

- show ip dhcp snooping

- → shows enabled VLANs, trusted/untrusted ports, option 82 status

- → verify ALL uplink ports show 'yes' under Trusted column

- → verify end device ports show 'no' under Trusted column

- show ip dhcp snooping binding

- → shows binding table: MAC + IP + lease + VLAN + interface

- → used by DAI (Lab 14) to validate ARP requests

- → if empty after DHCP — snooping may be blocking offers

- show ip dhcp snooping statistics

- → shows dropped packets count

- → high drop count = rogue DHCP detected or misconfiguration

## 7G. Troubleshooting DHCP Snooping

- Symptom: PCs getting APIPA (169.254.x.x) after snooping enabled

- Cause 1: Uplink port not marked as trusted — DHCP offers being dropped

- Fix 1: ip dhcp snooping trust on ALL uplink ports including port-channel

- Cause 2: Option 82 mismatch between switch and DHCP server

- Fix 2: no ip dhcp snooping information option on all switches

- Cause 3: VLAN not included in snooping config

- Fix 3: ip dhcp snooping vlan [missing VLAN]

- Cause 4: PT limitation with EtherChannel

- Fix 4: In PT — remove snooping, document limitation

- In production — configure trust on port-channel interface

- Golden rule: if DHCP breaks after snooping — check trusted ports first

## 7H. Packet Tracer Limitation — Documented

- Issue: port-channel interface does not accept ip dhcp snooping trust in PT

- Impact: DHCP offers blocked even with physical uplinks trusted

- Result: PCs fall back to APIPA — DHCP fails

- Workaround in PT: remove ip dhcp snooping — DHCP works without it

- Production reality: this works correctly on real Cisco hardware

- Documentation note: DHCP Snooping fully understood and designed

- PT limitation does not reflect production capability

- This is honest professional documentation — real engineers document limitations

## 7I. DHCP Snooping Binding Table — Key for Lab 14

- DHCP Snooping creates a binding table: IP + MAC + VLAN + Port

- This table is used by Dynamic ARP Inspection (DAI) in Lab 14

- DAI checks ARP requests against snooping binding table

- If ARP IP/MAC doesn't match binding table = drop = ARP spoofing prevented

- In PT: since snooping not running, DAI will use static ARP entries instead

- In production: always enable snooping BEFORE DAI

- Order: DHCP Snooping first → binding table populated → DAI enabled

## 8. ISO 27001 & NIST Links

- ISO 27001 → A.9.1 Access Control Policy — DHCP Snooping enforces network access control

- ISO 27001 → A.13.1 Network Controls — prevents rogue devices from operating

- ISO 27001 → A.12.4 Logging — snooping statistics provide audit trail

- NIST CSF  → PR.AC-3 Remote access managed — unauthorised DHCP servers blocked

- NIST CSF  → PR.AC-5 Network integrity — trust model enforced at Layer 2

- NIST CSF  → DE.CM-1 Network monitored — snooping statistics alert on attacks

## 9. Bank Relevance

- Bangladesh Bank ICT Guidelines classify rogue DHCP as critical Layer 2 threat

- Man in the Middle via rogue DHCP = banking credential theft = catastrophic

- DHCP Snooping mandatory in all Bangladesh Bank regulated institutions

- Snooping binding table supports forensic investigation after incidents

- Any port with untrusted DHCP activity = immediate alert to SOC team

- Real banks combine: DHCP Snooping + DAI + 802.1X for complete Layer 2 security

NexaBank Enterprise Network Project | Asfi Ahmed | GitHub: NexaBank-Enterprise-Network