╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 09

LAB TITLE     : NAT & Internet Access (PAT)

DATE          : Today

SCENARIO      : NexaBank HO — 500 employees

accessing internet through

single public IP on R1

WHY WE DID IT : Internal 10.1.x.x addresses

are private — internet doesn't

know how to route to them.

PAT translates all internal

IPs to R1's single public IP.

Hides internal topology from

internet — security + efficiency.

WHAT WE BUILT : PAT configured on R1.

Inside interface = Gig0/0/0

Outside interface = Gig0/0/1

ACL permits all 10.1.x.x traffic

All 5 VLANs tested — 100% success

KEY COMMANDS  :

ip nat inside

ip nat outside

access-list 1 permit 10.1.0.0 0.0.255.255

ip nat inside source list 1

interface Gig0/0/1 overload

show ip nat translations

WHAT WE LEARNED:

→ NAT = translates private to public IP

→ PAT = many devices share ONE public IP

→ PAT also called NAT Overload

→ Wildcard mask = inverse of subnet mask

→ Inside local = real private IP

→ Inside global = public IP ISP sees

→ Port numbers differentiate PAT sessions

→ ACL defines which IPs get translated

WHAT BROKE    : Nothing this lab

WHAT SAVED IT : N/A

REAL LIFE ACTION: In production, public IP

is assigned by ISP — not

10.9.1.1. Real bank gets

dedicated public IP block

from ISP for NAT pool.

ISO 27001 LINK: A.13.1 — Network Controls

NAT hides internal topology

Protects network architecture

from external exposure

NIST CSF LINK : PR.AC-5 — Network integrity

DE.CM-1 — Network monitored

NAT translations logged

for security monitoring

BANK RELEVANCE: Bangladesh Bank ICT Guidelines

require internal network not

exposed to public internet.

NAT/PAT = first line of defense.

Hides ATM gateway, core banking

servers from direct attack.

TEST RESULT   : PAT configured on R1 ✅

All 5 VLANs → ISP 4/4 ✅

NAT translations verified ✅

Internal IPs hidden from ISP ✅

LAB 09 — NAT & INTERNET ACCESS (PAT)

======================================

1. WHY NAT EXISTS

──────────────────

Private IPs (10.x.x.x) not routable

on public internet.

ISP has no idea where 10.1.10.2 is.

NAT bridges this gap.

Without NAT:

→ Internal devices can't reach internet

→ Private IPs dropped at ISP ❌

With NAT:

→ R1 translates private to public

→ ISP only sees R1's public IP ✅

2. NAT TYPES

─────────────

Static NAT  → one private = one public

1:1 mapping

expensive, not scalable

Dynamic NAT → pool of public IPs

assigned dynamically

still needs multiple public IPs

PAT         → many private = ONE public

(NAT Overload) differentiated by port number

most common in enterprise

NexaBank uses this ✅

3. HOW PAT WORKS

─────────────────

PC1 (10.1.10.2) sends packet to internet

R1 receives it → checks NAT rules

R1 translates:

→ Source IP: 10.1.10.2 → 10.9.1.1

→ Source Port: 1024 (unique per session)

ISP receives packet from 10.9.1.1:1024

ISP sends reply to 10.9.1.1:1024

R1 receives reply → checks translation table

R1 translates back:

→ Destination: 10.9.1.1:1024 → 10.1.10.2

PC1 receives reply ✅

500 devices differentiated by port number only.

4. WILDCARD MASK

─────────────────

Opposite of subnet mask

Used in ACLs, NAT, OSPF

0 = must match exactly

1 = don't care

Quick flip trick:

255 → 0

0   → 255

224 → 31

192 → 63

128 → 127

Example:

10.1.0.0 0.0.255.255

= match any 10.1.x.x address

Calculation:

255.255.255.255

- 255.255.0.0 (subnet)

= 0.0.255.255 (wildcard)

5. NAT CONFIGURATION ON R1

────────────────────────────

Step 1 — Define inside/outside interfaces:

int Gig0/0/0

ip nat inside    → LAN side

int Gig0/0/1

ip nat outside   → WAN side

Step 2 — Define which IPs get translated:

access-list 1 permit 10.1.0.0 0.0.255.255

→ permit all 10.1.x.x addresses

Step 3 — Enable PAT:

ip nat inside source list 1

interface Gig0/0/1 overload

→ overload = PAT (many to one)

6. NAT TRANSLATION TABLE

──────────────────────────

show ip nat translations

Columns:

Inside local  = real private IP (10.1.x.x)

Inside global = public IP ISP sees (10.9.1.1)

Outside local = destination as seen from inside

Outside global= destination real public IP

Port after colon = PAT port number

Each session gets unique port number

7. SECURITY BENEFITS

──────────────────────

→ Internal topology hidden from internet

→ Attackers can't directly target

internal devices

→ Finance, Teller, ATM systems protected

→ Bangladesh Bank ICT Guidelines compliant

→ First line of network defense

8. ISO 27001 & NIST LINKS

───────────────────────────

ISO 27001 → A.13.1 Network Controls

NAT hides internal network

architecture from public

NIST CSF  → PR.AC-5 Network integrity

DE.CM-1 Network monitored

NAT translations logged

9. BANK RELEVANCE

──────────────────

Bangladesh Bank requires:

→ Internal network not exposed publicly

→ All internet traffic through controlled

gateway (R1)

→ NAT translations logged for audit

→ Single exit point for monitoring

Real world difference:

Lab    → 10.9.1.1 as public IP

Prod   → Real public IP from ISP

e.g. 103.x.x.x for Bangladesh ISPs

10. WHAT BROKE / SAVED / REAL LIFE

─────────────────────────────────────

WHAT BROKE:

→ Nothing this lab

WHAT SAVED IT:

→ N/A

REAL LIFE ACTION:

→ Production uses real public IP from ISP

→ NAT logs monitored by NOC team daily

→ Unusual translation volumes = alert

→ Could indicate data exfiltration attempt