LAB NUMBER : 07
LAB TITLE : LACP EtherChannel Implementation (Port Aggregation)
DATE : Today
SCENARIO : NexaBank HO Core-to-Access Link Aggregation
between CS1 core switch and AS1/AS2/AS3 access switches

## WHY WE DID IT :

Single physical links between core and access switches create:

- bandwidth bottlenecks

- STP blocking of redundant links

- single link failure risk

EtherChannel using LACP solves this by:

- bundling multiple physical links into one logical interface

- increasing bandwidth

- providing automatic redundancy without STP blocking

Critical for bank backbone links where uptime and throughput matter.

## WHAT WE BUILT :

- CS1 connected to AS1, AS2, AS3 using dual uplinks each

- Each switch pair configured into LACP EtherChannel:

  - CS1 ↔ AS1 = Port-Channel 1

  - CS1 ↔ AS2 = Port-Channel 2

  - CS1 ↔ AS3 = Port-Channel 3

- All links aggregated using LACP (mode active)

## KEY COMMANDS :

interface range gig1/0/x , gig1/0/y
channel-group [id] mode active

show etherchannel summary
show spanning-tree
show cdp neighbors
show interfaces status

## WHAT WE LEARNED :

→ EtherChannel groups multiple physical links into one logical link
→ LACP (IEEE standard) enables dynamic link negotiation
→ STP treats Port-Channel as a single interface
→ Prevents STP blocking on redundant uplinks
→ Improves bandwidth utilization and fault tolerance
→ Link failure does NOT break connectivity
→ Only reduces bandwidth temporarily
→ Core switch becomes aggregation point for multiple branches

## TROUBLESHOOTING :

→ Initial STP warnings during EtherChannel formation
→ Port state changes (up/down flaps) during LACP negotiation
→ Standalone interfaces (I state) when remote side not configured
→ Fixed by ensuring both ends use LACP mode active

## ISO 27001 LINK :

A.17.1 — Information processing facilities availability management
EtherChannel ensures continuous network availability by eliminating single link dependency.

## NIST CSF LINK :

PR.PT-5 — Resilience mechanisms implemented
LACP provides redundancy and failover at Layer 2 infrastructure level.

## BANK RELEVANCE :

In banking environments (UCB, DBBL, GP backbones):

- Core-to-access uplinks use EtherChannel

- Ensures uninterrupted transaction processing

- Prevents downtime during cable failures

- Supports high-volume data traffic between branches and core systems

## TEST RESULT :

- EtherChannel successfully formed on CS1, AS1, AS2, AS3 ✅

- All Port-Channels in SU state ✅

- All member links in P state (active bundling) ✅

- STP recognizes logical interfaces correctly ✅

- Single link failure test passed without downtime ✅

- Network fully stable after recovery test ✅

# 📘 LAB 07 — LACP ETHERCHANNEL (DETAILED NOTES)

## 🧠 1. Objective

To implement Link Aggregation Control Protocol (LACP) between core and access switches in a multi-switch enterprise topology, improving bandwidth, redundancy, and STP efficiency.

## 🧱 2. Topology Summary

- Core Switch: CS1 (Catalyst 3650)

- Access Switches: AS1, AS2, AS3 (Catalyst 2960)

- Links per switch: 2 physical uplinks each

### Logical Design:

- CS1 ↔ AS1 = Port-Channel 1

- CS1 ↔ AS2 = Port-Channel 2

- CS1 ↔ AS3 = Port-Channel 3

## 🔌 3. Technology Used

### LACP (Link Aggregation Control Protocol)

- IEEE 802.3ad standard

- Bundles multiple physical links into one logical interface

- Supports active/active redundancy

- Automatically detects link failures

## ⚙️ 4. Configuration Summary

### CS1 Configuration

conf t

interface range gig1/0/1 , gig1/0/4
channel-group 1 mode active

interface range gig1/0/2 , gig1/0/5
channel-group 2 mode active

interface range gig1/0/3 , gig1/0/6
channel-group 3 mode active

end

### AS1 Configuration

conf t

interface range gig0/1 , gig0/2
channel-group 1 mode active

end

### AS2 Configuration

conf t

interface range gig0/1 , gig0/2
channel-group 2 mode active

end

### AS3 Configuration

conf t

interface range gig0/1 , gig0/2
channel-group 3 mode active

end

## 🔍 5. Verification Commands

show etherchannel summary
show spanning-tree
show interfaces trunk
show cdp neighbors

## 📊 6. EtherChannel States

| State | Meaning |
| --- | --- |
| SU | Layer 2, in use (correct) |
| P | Port successfully bundled |
| I | Standalone (not bundled) |
| D | Down |

## 🧠 7. STP Behaviour Change

Before EtherChannel:

- STP blocks redundant links

- Multiple physical ports visible

After EtherChannel:

- STP sees ONE logical interface (Port-Channel)

- No blocking on member links

- All links actively used

## 😈 8. Failure Test (Validation)

### Action:

- Shut down Gig1/0/1 on CS1

### Result:

- Link removed from bundle

- Port-Channel remains UP

- Traffic continues via remaining link

- No STP recalculation triggered

## 🧾 9. Key Learning Outcomes

- EtherChannel aggregates bandwidth across multiple links

- LACP ensures dynamic failure handling

- STP treats Port-Channel as a single interface

- Network becomes resilient and loop-free

- Eliminates wasted redundant links caused by STP blocking