LAB 15
 802.1X Port Authentication

# Lab Documentation Card

LAB NUMBER: 15

LAB TITLE: 802.1X Port Authentication

DATE: 29 May 2026

SCENARIO: NexaBank HO — implementing identity-based network access control. No device gets network access without authenticating against RADIUS server, regardless of physical connection.

WHY WE DID IT: Port Security only checks MAC address — easily spoofed. 802.1X checks actual identity (username/password or certificate) verified by a central RADIUS server. A cleaner plugging in after hours gets zero access — even physically connected.

WHAT WE BUILT: RADIUS-SVR configured with user accounts and shared secret. 802.1X concept fully demonstrated. Production config documented. PT limitation noted.

WHAT BROKE: Cat3650 in PT does not support AAA commands (aaa new-model, aaa authentication dot1x). dot1x show commands also not available. Full PT limitation.

WHAT SAVED IT: Documented limitation professionally. Full production config available for real hardware deployment.

REAL LIFE ACTION: On real Cisco hardware: configure aaa new-model, point to RADIUS server, enable dot1x on access ports. Works fully on Cat2960/3650 in production.

ISO 27001 LINK: A.9.1.2 — Access to Networks and Network Services | A.9.4.2 — Secure Log-on Procedures | 802.1X enforces identity-based network access

NIST CSF LINK: PR.AC-1 — Identities managed | PR.AC-3 — Remote access managed | PR.AC-7 — Users authenticated

BANK RELEVANCE: Bangladesh Bank ICT Guidelines require identity-based access control for network infrastructure. 802.1X prevents unauthorised physical access — cleaners, visitors, contractors cannot plug in and get access.

TEST RESULT: RADIUS-SVR configured ✅ | User accounts created ✅ | Shared secret configured ✅ | Production config documented ✅ | PT limitation noted ✅

# Key Commands — 802.1X (Production)

- aaa new-model                                    → enable AAA framework

- aaa authentication dot1x default group radius    → use RADIUS for 802.1X auth

- dot1x system-auth-control                        → enable 802.1X globally

- radius-server host [IP] auth-port 1812 key [key] → point to RADIUS server

- interface [port]                                 → enter access port

- dot1x port-control auto                          → enable 802.1X on port

- authentication port-control auto                 → newer IOS syntax

- show dot1x                                       → verify 802.1X status

- show dot1x interface [port]                      → per port 802.1X status

- show authentication sessions                     → view authenticated sessions

# Notes

## 1. What is 802.1X

802.1X is an IEEE standard for port-based network access control. It provides an authentication mechanism to devices wishing to connect to a network. Unlike Port Security which checks MAC addresses, 802.1X verifies actual identity through a central authentication server before allowing ANY network access — even IP assignment is blocked until authentication succeeds.

## 2. The Three Players

- Supplicant   = device trying to connect (PC, laptop, phone)

- = must have 802.1X client software (built into Windows/Linux)

- Authenticator = switch enforcing access control (AS1, AS2, AS3)

- = blocks port until RADIUS says YES

- Auth Server  = RADIUS server verifying identity (RADIUS-SVR)

- = checks username/password or certificate

- = sends Accept or Reject back to switch

## 3. 802.1X Authentication Flow

- Step 1 — Device plugs into switch port

- Step 2 — Switch port in UNAUTHORISED state — only EAP traffic allowed

- Step 3 — Switch sends EAP-Request/Identity to device

- Step 4 — Device sends EAP-Response with username

- Step 5 — Switch forwards to RADIUS server

- Step 6 — RADIUS server challenges device (password/certificate)

- Step 7 — Device responds with credentials

- Step 8 — RADIUS sends Access-Accept or Access-Reject

- Step 9 — If Accept: switch port moves to AUTHORISED state

- Device gets DHCP IP and full network access ✅

- Step 10 — If Reject: port stays UNAUTHORISED — zero access ❌

## 4. Port Security vs 802.1X

- Port Security  → checks MAC address only

- → MAC spoofing bypasses it easily

- → no server needed

- → simple but weak

- 802.1X         → checks actual identity (username/password/certificate)

- → cannot spoof identity without credentials

- → requires RADIUS server

- → strong, enterprise grade

- Banks use BOTH together:

- Port Security = who is physically there (MAC level)

- 802.1X        = who are you actually (identity level)

## 5. RADIUS Server

- RADIUS = Remote Authentication Dial-In User Service

- Centralised authentication server for entire network

- All switches point to same RADIUS server

- One place to manage all user accounts — add/remove/disable

- Shared secret = password between switch and RADIUS server

- Auth port 1812 = standard RADIUS authentication port

- NexaBank RADIUS-SVR IP: 10.1.20.2 (IT VLAN — IT manages it)

## 6. TACACS+ vs RADIUS

- RADIUS:

- → Open standard (RFC 2865)

- → UDP based

- → Encrypts only password in packet

- → Used for network access (802.1X)

- → Supported by all vendors

- TACACS+ (Cisco proprietary):

- → Cisco proprietary protocol

- → TCP based (more reliable)

- → Encrypts entire packet

- → Used for device management (SSH login to router)

- → Cisco devices only

- NexaBank uses RADIUS for 802.1X (network access)

- Real banks often use TACACS+ for device management access

## 7. Complete Production Configuration

- On CS1 (and all switches):

- aaa new-model

- aaa authentication dot1x default group radius

- dot1x system-auth-control

- radius-server host 10.1.20.2 auth-port 1812 key NexaBank123

- On each access port (AS1/AS2/AS3 Fa0/1-24):

- interface range FastEthernet0/1-24

- dot1x port-control auto

- exit

- On RADIUS-SVR (Packet Tracer AAA service):

- Add client: CS1 IP, shared secret NexaBank123

- Add user: nexabank-user / NexaBank@123

- Auth port: 1812 (or 1645 in PT)

- Verify:

- show dot1x

- show dot1x interface fa0/1

- show authentication sessions

## 8. PT Limitation Note

- aaa new-model not supported on Cat3650 in PT

- aaa authentication dot1x command not available

- dot1x show commands not available

- All limitations are PT-specific — fully supported on real Cisco hardware

- RADIUS-SVR configured correctly — ready for production deployment

## 9. Bank Relevance

- Bangladesh Bank ICT Guidelines require identity-based network access control

- Cleaners, contractors, visitors = zero network access without credentials

- Central RADIUS = one place to disable compromised accounts immediately

- If employee is terminated: disable account in RADIUS = instant network lockout

- Audit trail: RADIUS logs every authentication attempt with timestamp

- 802.1X + Port Security + DHCP Snooping + DAI = complete Layer 2 security

POP QUIZ — Labs 01-15 Review (10 Questions)

Instructions: Answer from memory first. Then check answers below. These are real UCB interview style questions.

## Questions

Q1: What is the difference between a trunk port and an access port? When do you use each?

Answer: Access port carries ONE VLAN only — used for end devices (PCs, printers). Trunk port carries ALL VLANs simultaneously — used between switches and between switch and router. In NexaBank: Fa0/1 on AS1 = access port (PC1 VLAN10). Gig0/1 on AS1 to CS1 = trunk port. Rule: access ports face devices, trunk ports face network infrastructure.

Q2: Explain what an SVI is and why it is needed on CS1.

Answer: SVI = Switched Virtual Interface. A virtual Layer 3 interface created per VLAN on CS1. Acts as the default gateway for that VLAN. When PC1 (VLAN10) sends traffic to a different VLAN, it goes to the SVI (10.1.10.1) first. Without SVIs, VLANs are isolated with no inter-VLAN routing. ip routing command must be enabled for SVIs to route traffic. Example: interface vlan 10 / ip address 10.1.10.1 255.255.255.224

Q3: What is OSPF and why did NexaBank replace static routing with it?

Answer: OSPF = Open Shortest Path First. A dynamic routing protocol where routers automatically discover neighbours, share routing information and calculate the shortest path. NexaBank replaced static routing because: static routes don't adapt to failures (traffic keeps going to dead link), manual updates required for every change, not scalable. With OSPF: if a link fails at 3am, routes update automatically in seconds — no engineer intervention needed. Key command: router ospf 1 / network [ip] [wildcard] area 0.

Q4: What is PAT and how does it differ from basic NAT? Which does NexaBank use?

Answer: NAT = Network Address Translation. Basic NAT = one private IP maps to one public IP (1:1). PAT = Port Address Translation (also called NAT Overload) = many private IPs share ONE public IP, differentiated by port numbers. NexaBank uses PAT because we have 500 employees — buying 500 public IPs is expensive and unnecessary. All 500 devices share R1's single public IP (10.9.1.1). Command: ip nat inside source list 1 interface Gig0/0/1 overload — 'overload' is what makes it PAT.

Q5: A PC is getting 169.254.x.x instead of a proper IP. What does this mean and what do you check?

Answer: 169.254.x.x = APIPA (Automatic Private IP Addressing). This means the PC tried DHCP but got no response. Possible causes: DHCP server down or not configured, DHCP pool exhausted, wrong VLAN assignment on port, DHCP Snooping blocking the offer, trunk port misconfigured. Check: show ip dhcp binding (is server giving IPs?), show vlan brief (is port in correct VLAN?), ping the DHCP server IP, check show ip dhcp pool for exhaustion.

Q6: What is RSTP and why must CS1 always be the Root Bridge?

Answer: RSTP = Rapid Spanning Tree Protocol. Prevents broadcast storms by detecting loops and blocking redundant links. Converges in 1-3 seconds vs STP's 30-50 seconds. CS1 must be Root Bridge because it is the most powerful, central device — all traffic flows through it. If an access switch becomes Root Bridge, traffic takes suboptimal paths causing high latency. We manually set CS1 priority to 4096 (default 32768) using: spanning-tree vlan 1,10,20,30,40,50,60 priority 4096. Lower priority = wins election.

Q7: What is EtherChannel and what problem does it solve?

Answer: EtherChannel bundles multiple physical cables into ONE logical interface. Solves two problems: bandwidth bottleneck (single cable limits throughput) and STP blocking (RSTP blocks redundant links wasting bandwidth). With EtherChannel, STP sees ONE logical link = no blocking, both cables carry traffic simultaneously. If one cable fails, traffic continues on the other. NexaBank uses LACP (IEEE standard) mode active. Command: interface range Gig1/0/1, Gig1/0/4 / channel-group 1 mode active.

Q8: What is the purpose of the 'no switchport' command on CS1's interface facing R1?

Answer: Cat3650 ports are Layer 2 by default — they switch frames but cannot have IP addresses. The link between CS1 and R1 is a routed point-to-point link needing its own IP address on each end. 'no switchport' converts the port from Layer 2 mode to Layer 3 mode, allowing an IP address to be assigned. Without this command, CS1 cannot communicate directly with R1 at Layer 3. Applied on: interface GigabitEthernet1/0/7 / no switchport / ip address 10.1.1.2 255.255.255.252.

Q9: Explain the difference between DHCP Snooping and DAI. Why must DHCP Snooping be configured before DAI?

Answer: DHCP Snooping: Layer 2 security that prevents rogue DHCP servers. Classifies ports as trusted/untrusted. Only trusted uplink ports can send DHCP offers. Builds a binding table: IP + MAC + VLAN + Port. DAI (Dynamic ARP Inspection): prevents ARP Spoofing/MitM attacks. Validates every ARP packet against the DHCP Snooping binding table. If ARP IP-MAC doesn't match binding table = drop. DHCP Snooping must come first because DAI needs the binding table to validate ARP. Without snooping = no binding table = DAI has no reference = cannot validate ARP = shows Inactive.

Q10: What is the DMZ and why can't NexaBank just put servers in their own VLAN instead?

Answer: DMZ = Demilitarised Zone. An isolated network zone between the untrusted internet and the trusted internal LAN. Hosts public-facing servers: WEB-SVR, ATM-GW, MAIL-SVR. A VLAN is logical segmentation INSIDE the internal network — still part of the trusted zone. One misconfigured ACL and an attacker reaches Finance directly. DMZ is architecturally separate — even if ACLs fail, DMZ is structurally isolated from internal LAN. Bangladesh Bank mandates proper DMZ architecture for banks running public services. Key principle: blast radius of a DMZ breach is contained to the public zone only — internal LAN stays safe.

LAB 15 BONUS — TACACS+ Device Management Authentication

WHAT IS TACACS+

TACACS+ (Terminal Access Controller Access Control System Plus) is a Cisco proprietary protocol used for centralised device management authentication. While 802.1X controls who can access the network, TACACS+ controls who can log into network devices (routers, switches, firewalls) via SSH.

TACACS+ vs RADIUS — Full Comparison

Feature          RADIUS              TACACS+

─────────────────────────────────────────────

Standard         Open (RFC 2865)     Cisco proprietary

Transport        UDP                 TCP (more reliable)

Encryption       Password only       Entire packet

Primary use      Network access      Device management

(802.1X)            (SSH/Telnet login)

Port             1812/1645           49

Vendor support   All vendors         Cisco only

Accounting       Basic               Detailed per command

HOW TACACS+ WORKS IN NEXABANK

IT engineer SSH → CS1

CS1 asks TACACS+ server:

"Is 'admin' with this password valid?"

TACACS+ checks user database

YES → SSH session opened ✅

NO  → Connection rejected ❌

Only IT staff with valid TACACS+ credentials can manage network devices. Even if someone knows the device IP, without TACACS+ credentials they cannot log in.

NEXABANK TACACS+ CONFIGURATION

RADIUS-SVR AAA Service settings:

Client Name : CS1

Client IP   : 10.1.20.1

Secret      : Tacacs123

Server Type : TACACS

Username    : admin

Password    : Admin@123

CS1 CLI configuration:

aaa new-model

tacacs-server host 10.1.20.2 key Tacacs123

aaa authentication login default group tacacs+ local

ip domain-name nexabank.local

crypto key generate rsa (1024 bits)

ip ssh version 2

username admin privilege 15 secret Admin@123

line vty 0 4

transport input ssh

login authentication default

exit

VERIFICATION

From PC1 command prompt:

ssh -l admin 10.1.20.1

Enter password: Admin@123 Result: CS1> prompt = TACACS+ authentication working ✅

KEY COMMANDS REFERENCE

aaa new-model                          → enable AAA

tacacs-server host [IP] key [secret]   → point to TACACS+ server

aaa authentication login default       → use TACACS+ for login

group tacacs+ local

show tacacs                            → verify TACACS+ config

show aaa sessions                      → view active sessions

debug tacacs                           → troubleshoot auth issues

WHY LOCAL FALLBACK

aaa authentication login default group tacacs+ local

The word local at the end = fallback to local username if TACACS+ server is unreachable.

Critical in banks — if RADIUS-SVR goes down, engineers can still access devices using local credentials. Without fallback = locked out of entire network during server outage. Always include local fallback.

BANK RELEVANCE

→ Every network device login goes through TACACS+

→ Centralised audit trail — who logged in, when, what commands

→ Instant account disable — terminated employee = disable in TACACS+

= locked out of ALL devices immediately

→ Bangladesh Bank ICT Guidelines require centralised

device management authentication

→ TACACS+ command accounting logs every command typed

= full audit trail for compliance

ISO 27001 LINKS

A.9.4.2 — Secure Log-on Procedures

A.9.4.4 — Use of Privileged Utility Programs

A.12.4.1 — Event Logging (TACACS+ logs all access)

NIST CSF LINKS

PR.AC-1 — Identities managed for authorised devices

PR.AC-4 — Access permissions managed

DE.CM-3 — Personnel activity monitored

WHAT BROKE / SAVED / REAL LIFE

WHAT BROKE   : 802.1X fully limited on Cat3650 in PT

WHAT WE DID  : Implemented TACACS+ as alternate

demonstration — equally important

real world security control

REAL LIFE    : Production banks run BOTH:

802.1X for network access control

TACACS+ for device management control

Together = complete access security

NexaBank Enterprise Network Project | Asfi Ahmed | GitHub: NexaBank-Enterprise-Network