╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 05

LAB TITLE     : Access Layer Switches

DATE          : Today

SCENARIO      : NexaBank HO access layer —

floor level switch configuration

with port security hardening

WHY WE DID IT : Access switches connect end devices

to the network. Port security

prevents unauthorised access —

critical in a bank environment

WHAT WE BUILT : AS1, AS2, AS3 fully configured

with VLANs, access ports and

port security on all ports

KEY COMMANDS  :

int range fa0/1-24

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

show port-security interface fa0/x

WHAT WE LEARNED:

→ Port mapping — assigning ports to VLANs

→ Interface range command saves time

→ Port security — sticky MAC, violation modes

→ Err-disabled state and how to fix it

→ Must set access mode before port security

ISO 27001 LINK: A.9.1 — Access Control Policy

A.11.2 — Equipment Security

Port security enforces physical

access control at network level

NIST CSF LINK : PR.AC-3 — Remote access managed

PR.PT-3 — Least functionality

applied to network ports

BANK RELEVANCE: Bangladesh Bank ICT Guidelines

require access control on all

network ports. Port security

prevents insider threats —

most common attack in banks.

Supports audit compliance.

TEST RESULT   : Port security enabled all switches

Sticky MAC learning active

Violation mode shutdown ✅

LAB 05 — ACCESS LAYER SWITCHES

================================

1. ACCESS SWITCH ROLE

──────────────────────

Connects end devices to network

Layer 2 only — no routing

One per floor or department

Assigns ports to correct VLANs

Reports up to core switch (CS1)

Real world:

→ 24 port switch per floor

→ Ports grouped by department

→ Port count per dept based on

number of devices on that floor

2. PORT MAPPING

────────────────

Assigning switch ports to VLANs

based on department device count

Example:

Fa0/1-8   → VLAN 10 Management

Fa0/9-16  → VLAN 20 IT

Fa0/17-20 → VLAN 40 HR

Fa0/21-24 → VLAN 50 Tellers

NexaBank HO port assignments:

AS1 Fa0/1 → VLAN 10 (PC1 Management)

AS1 Fa0/2 → VLAN 40 (PC4 HR)

AS2 Fa0/1 → VLAN 20 (PC2 IT)

AS2 Fa0/2 → VLAN 50 (PC5 Tellers)

AS3 Fa0/1 → VLAN 30 (PC3 Finance)

3. INTERFACE RANGE COMMAND

────────────────────────────

Configures multiple ports simultaneously

Saves time on large switches

int range fa0/1-24

→ applies commands to all 24 ports at once

Rule: Always set switchport mode access

BEFORE applying port security

Dynamic ports reject port security

4. PORT SECURITY

─────────────────

Prevents unauthorised devices

from connecting to switch ports

How it works:

→ Each port allows only approved MAC

→ Unknown MAC = violation = action taken

→ Protects against physical intrusion

Two MAC learning methods:

Static → manually type MAC address

time consuming, not scalable

Sticky → auto learns first device MAC

saves permanently

used in real bank networks ✅

Three violation modes:

Shutdown  → port goes err-disabled ✅ (banks use this)

Restrict  → drops traffic, logs violation

Protect   → drops traffic, no logging

5. PORT SECURITY COMMANDS

──────────────────────────

Enable port security:

switchport port-security

Set maximum devices:

switchport port-security maximum 1

Set sticky MAC learning:

switchport port-security mac-address sticky

Set violation mode:

switchport port-security violation shutdown

Verify:

show port-security interface fa0/x

show port-security address

6. ERR-DISABLED STATE

──────────────────────

Happens when wrong device connects

after MAC was already learned

Example:

→ PC connected → MAC learned

→ Different device plugged in

→ Different MAC = violation

→ Port shuts down = err-disabled

Fix:

int fa0/x

shutdown

no shutdown

→ Must do shutdown first to reset port

7. PC IP CONFIGURATION

────────────────────────

PC1 VLAN 10 Management:

IP: 10.1.10.2  Mask: 255.255.255.224  GW: 10.1.10.1

PC2 VLAN 20 IT:

IP: 10.1.20.2  Mask: 255.255.255.192  GW: 10.1.20.1

PC3 VLAN 30 Finance:

IP: 10.1.30.2  Mask: 255.255.255.128  GW: 10.1.30.1

PC4 VLAN 40 HR:

IP: 10.1.40.2  Mask: 255.255.255.224  GW: 10.1.40.1

PC5 VLAN 50 Tellers:

IP: 10.1.50.2  Mask: 255.255.255.192  GW: 10.1.50.1

8. ISO 27001 & NIST LINKS

───────────────────────────

ISO 27001 → A.9.1 Access Control Policy

A.11.2 Equipment Security

Port security enforces physical

access control at network level

NIST CSF  → PR.AC-3 Remote access managed

PR.PT-3 Least functionality

applied to network ports

9. BANK RELEVANCE

──────────────────

Bangladesh Bank ICT Guidelines require:

→ Access control on all network ports

→ Unauthorised device prevention

→ Physical security at network level

→ Documented port security policy

Insider threats = most common attack

in financial institutions worldwide.

Port security directly mitigates this.