╔══════════════════════════════════════════╗

║         LAB DOCUMENTATION CARD          ║

╚══════════════════════════════════════════╝

LAB NUMBER    : 04

LAB TITLE     : Core Switch Configuration

DATE          : Today

SCENARIO      : NexaBank HO core switching layer

CS1 configured as Layer 3 backbone

WHY WE DID IT : Core switch is the backbone of HO

All VLANs and inter-dept traffic

flows through CS1

WHAT WE BUILT : CS1 Layer 3 core switch with

6 VLANs, trunk ports to 3 access

switches, SVIs for inter-VLAN routing

KEY COMMANDS  :

vlan [id] / name [name]

switchport mode trunk

interface vlan [id]

ip address [ip] [mask]

no shutdown

ip routing

show vlan brief

show interfaces trunk

show ip interface brief

WHAT WE LEARNED:

→ VLAN creation and naming

→ Trunk port configuration

→ SVI creation for inter-VLAN routing

→ ip routing command activates Layer 3

→ Default gateway = first usable IP of subnet

ISO 27001 LINK: A.13.1 Network Controls

VLAN segmentation enforces

network access control

NIST CSF LINK : PR.AC-5 Network integrity

protected through segmentation

BANK RELEVANCE: Bangladesh Bank requires network

segmentation between departments

VLANs prevent lateral movement

of threats across bank network

TEST RESULT   : Inter-VLAN ping PC1→PC2→PC3 ✅

0% packet loss across all VLANs ✅

LAB 04 — CORE SWITCH CONFIGURATION

=====================================

STEP 1 — Basic Setup

enable              → enter privileged mode

conf t              → enter configuration mode

hostname CS1        → name the switch CS1

STEP 2 — Create VLANs

vlan 10             → create VLAN 10

name Management     → name it Management

(repeat for 20,30,40,50,60)

→ tells switch these VLANs exist

STEP 3 — Trunk Ports (CS1 to Access Switches)

interface Gig1/0/1  → go into port facing AS1

switchport mode trunk → carry all VLANs on this link

(repeat for Gig1/0/2, Gig1/0/3)

→ trunk = highway carrying all VLANs together

STEP 4 — Create SVIs (Default Gateways)

interface vlan 10         → create virtual interface

ip address 10.1.10.1 255.255.255.224 → assign IP

no shutdown               → turn it on

(repeat for all 6 VLANs)

→ SVI acts as default gateway for each VLAN

STEP 5 — Enable IP Routing

ip routing

→ activates Layer 3 on the switch

→ without this, SVIs exist but cant route

STEP 6 — Access Switch Setup (AS1, AS2, AS3)

hostname AS1/AS2/AS3      → name each switch

vlan 10-60                → create same VLANs

interface Gig0/1          → port facing CS1

switchport mode trunk     → trunk to CS1

interface Fa0/1           → port facing PC

switchport mode access    → single VLAN only

switchport access vlan 10 → assign to VLAN

→ access port = end device connection

STEP 7 — Assign PC IPs

IP address  → first usable IP in subnet

Subnet mask → matching subnet mask

Gateway     → SVI address on CS1

→ gateway tells PC where to send

traffic destined outside its VLAN

VERIFICATION COMMANDS:

show vlan brief           → confirm VLANs exist

show interfaces trunk     → confirm trunk ports

show ip interface brief   → confirm SVIs are up

ping [ip]                 → test connectivity

LAB 04 — CORE SWITCH CONFIGURATION

=====================================

1. WHAT IS A CORE SWITCH

──────────────────────────

Layer 3 switch — backbone of the network

All department traffic flows through it

Handles both switching (L2) and routing (L3)

NexaBank device: Cisco Catalyst 3650

2. VLAN RECAP

──────────────

VLAN = Virtual LAN = gated community

Segments network into separate broadcast domains

Devices in same VLAN → talk freely

Devices in different VLAN → blocked unless router allows

Without VLAN:

→ All departments share same broadcast domain

→ Virus in Teller floor spreads to Finance instantly

→ Security nightmare for a bank ❌

With VLAN:

→ Each department isolated

→ Threat contained to one VLAN

→ Access controlled by router ✅

NexaBank VLANs:

VLAN 10 = Management  → 10.1.10.0/27

VLAN 20 = IT          → 10.1.20.0/26

VLAN 30 = Finance     → 10.1.30.0/25

VLAN 40 = HR          → 10.1.40.0/27

VLAN 50 = Tellers     → 10.1.50.0/26

VLAN 60 = DMZ         → 10.1.60.0/28

3. PORT TYPES

──────────────

Access port → connects end devices (PCs, printers)

carries ONE VLAN only

used on access switches

Trunk port  → connects switch to switch

or switch to router

carries ALL VLANs together

used between CS1 and AS1/AS2/AS3

4. SVI — SWITCHED VIRTUAL INTERFACE

─────────────────────────────────────

Virtual interface created per VLAN on Layer 3 switch

Acts as default gateway for that VLAN

All traffic leaving a VLAN passes through SVI

Requires 'ip routing' command to work

NexaBank SVI addresses (default gateways):

VLAN 10 → 10.1.10.1  (Management gateway)

VLAN 20 → 10.1.20.1  (IT gateway)

VLAN 30 → 10.1.30.1  (Finance gateway)

VLAN 40 → 10.1.40.1  (HR gateway)

VLAN 50 → 10.1.50.1  (Tellers gateway)

VLAN 60 → 10.1.60.1  (DMZ gateway)

Rule: SVI address = first usable IP of subnet

5. INTER-VLAN ROUTING

───────────────────────

Traffic between VLANs must go through Layer 3

CS1 handles this via SVIs + ip routing command

Without ip routing → SVIs exist but cant route

Flow example:

PC1 (VLAN 10) → CS1 SVI → PC2 (VLAN 20)

Management    → gateway  → IT department

6. CONFIGURATION COMMANDS

───────────────────────────

Basic setup:

enable                    → privileged mode

conf t                    → config mode

hostname [name]           → name the device

VLAN creation:

vlan [id]                 → create VLAN

name [name]               → name the VLAN

Trunk port:

interface [port]          → enter port

switchport mode trunk     → set as trunk

SVI creation:

interface vlan [id]       → create SVI

ip address [ip] [mask]    → assign IP

no shutdown               → turn on

Routing:

ip routing                → enable Layer 3

Access port:

interface [port]          → enter port

switchport mode access    → set as access

switchport access vlan [id] → assign VLAN

7. VERIFICATION COMMANDS

──────────────────────────

show vlan brief           → confirm VLANs exist

show interfaces trunk     → confirm trunk ports

show ip interface brief   → confirm SVIs up/up

ping [ip]                 → test connectivity

8. PC IP CONFIGURATION

────────────────────────

PC1 (VLAN 10 Management):

IP      → 10.1.10.2

Mask    → 255.255.255.224

Gateway → 10.1.10.1

PC2 (VLAN 20 IT):

IP      → 10.1.20.2

Mask    → 255.255.255.192

Gateway → 10.1.20.1

PC3 (VLAN 30 Finance):

IP      → 10.1.30.2

Mask    → 255.255.255.128

Gateway → 10.1.30.1

9. TEST RESULTS

────────────────

PC1 → PC2 (VLAN 10 to VLAN 20) = 4/4 ✅

PC1 → PC3 (VLAN 10 to VLAN 30) = 4/4 ✅

PC1 → CS1 SVI (10.1.10.1)      = 4/4 ✅

10. ISO 27001 & NIST LINKS

────────────────────────────

ISO 27001 → A.13.1 Network Controls

VLAN segmentation enforces

network access control policy

NIST CSF  → PR.AC-5 Network integrity

protected through segmentation

11. BANK RELEVANCE

───────────────────

Bangladesh Bank requires:

→ Network segmentation between departments

→ Finance isolated from general staff network

→ Teller systems on separate VLAN

→ VLANs prevent lateral movement of threats

→ Documented VLAN policy for compliance audits

12. SPANNING TREE NOTE

────────────────────────

STP/RSTP runs automatically on switches

New trunk ports may take time to converge

If port shows 'none' in forwarding state

→ wait 30 seconds and check again

→ covered in detail in Lab 06